EV Charger

Product Catalogue

Keep Forging Ahead with Boundless Energy

EV CHARGER PRODUCT CATALOGUE

P04

EXP30K2

30kW DC Charging Power with Two DC Charging Connectors

+ Optional 22kW AC Type 2 Charging Socket

P10

EXP480K6

480kW DC Charging Power with Two DC Charging Connectors

P06

EXP60K3

60kW DC Charging Power with Two DC Charging Connectors

+ Optional 22kW AC Type 2 Charging Cable

P12

EXP300H9

180kW/240kW DC Charging Power with Two DC Charging Connectors + Optional 22kW AC Type 2 Charging Socket

P08

EXP180K2, EXP240K5

180kW/240kW DC Charging Power with Two DC Charging Connectors + Optional 22kW AC Type 2 Charging Socket

P14

EXP600K3

600kW Power Cube/12 Charging Connectors/Overhead Charging Dispenser Solution

P16

EXP800K6

800kW Power Cube/12 Charging Connectors/MCS Dispenser/ Overhead Charging Dispenser Solution

P18

IHEBS470K7D-350LL3C

400kW Charging Power/6 Charging Connectors/350kWh Split Type Battery Buffer Charger

P20

G3 Charging Dispenser

200A/250A/300A/375A/380A/

500A/600A Charging Connector

P22

MCS Charging Dispenser

200A/250A/300A/375A/380A

/500A/600A/1500A Charging Connector

P24

P26

V2G Charging Solution

22kW/44kW/66kW/132kW V2G DC Fast Charger

IHEBV470K7D-350LL3C

400kW Charging Power/2 Charging Connectors/350kWh All-in-One Battery Buffer Charger

POS machine

can be installed

Introduction

EXP30K2 DC charger is specifically designed for wall-mount, pedestal and movable installations, and is fully compatible with CCS/NACS/CHAdeMO/GBT EV charging standards. It features a small size and light weight, making it easy to install and relocate, along with a user-friendly interface, IP54 protection, and a robust, durable design for various outdoor applications. It can deliver up to 30kW charging power and 150Vdc to 1000Vdc charging voltage. It can be configured with an optional 22kW/32A AC Type 2 charging socket and portable accessories.

Main Features

Flexible installation options: wall-mount/pedestal

installation, portable cart installation

CCS1/CCS2/GBT/CHAdeMO/NACS support, AC Type 2 charging socket support

WEB/APP Maintenance Backend System enhances     the operation and maintenance effectiveness

TUV CE, cTUVus, RED, CB, OCA, FCC, CTEP and

Energy Star certification

Optional cable retractor/credit card payment

(POS), VIN code support BABA support

Operating temperature	-30°C~+65°C, full power output below 45°C, power derating 5%/°C above 45°C

Transportation/Storage

temperature	-40°C~+75°C

Relative humidity	≤95%RH, non-condensing

Altitude	2000m

Atmospheric pressure	80kPa~110kPa

Cooling method	Air cooling

Protection rating	IP54/IK10 (IK08 for touch screen)

Input Parameters

AC frequency	45Hz~65Hz

AC voltage	CE: 3-phase+N+PE, 380/400Vac±10%; UL: 3-phase+PE, 480Vac±10%

Max input current	CE: 48A+32A (for AC output); UL: 38A

Power factor	>0.99

THDi	<3% (rated input)

Output Parameters

Max DC output power	30kW

DC output voltage	150Vdc~1000Vdc Constant power voltage range	300Vdc~1000Vdc Max DC output current	100A

Optional AC charging output	AC charging socket 32A/3P/22kW

Performance

Max system efficiency	>95% (rated input and output)

Standby power	CE: 48W; UL: 42W

Noise	≤80dB

Standard

Backend	OCPP 1.6J, support firmware update to 2.0x, OTA/PnC/smart charging/power management support

CCS PLC communication	DIN70121 and ISOI5118

CHAdeMO	CHAdeMO V1.2

GBT	GB/T 18487.1-2015, GB/T 27930-2015, GB/T 20234.1~3-2015 TUV CE, cTUVus, RED, CB, OCA, FCC, CTEP, Energy Star

Certification

UL 2202/CSA C22.2; EN IEC 61851-21-2 Class B; EN IEC 61851-1/EN IEC 61851-23/EN 61851-24

Manufacture	BABA support

Function and Interface

HMI	Tempered glass protective 7" TFT touch screen LCD, 3 * RGB LED, EPO

Metering	DC energy meter, DL/T645 standard, optional MID-certified meter

Monitoring interface	LAN and LTE wireless modem support

Charging mode	Auto/By Time/By Plan/By kWh/By SOC

Power switching mode	Two DC charging connectors charging in sequence, AC socket/cable charging in parallel Payment	RFID, Mobile APP, POS (optional)

MBE function support	WEB/APP Maintenance Backend System enhances the operation and maintenance effectiveness

Charging Protection

Overcurrent protection, short circuit protection, overvoltage protection, undervoltage protection

Insulation monitoring, protective earth connection monitoring Reverse polarity protection, overtemperature protection RCDB for AC charging socket/cable protection

Configuration

Charging module	REG1K0100G/U 30kW 1000V power module

Charging connector	One or two CCS/NACS/GBT connectors, CHAdeMO + CCS/NACS/GBT connectors, optional AC Type 2 charging socket

Charging connector capacity	GBT: 125A; CCS: 125A; NACS: 125A; CHAdeMO: 125A; AC: 32A/22kW

Optional pedestal	Pedestal with two cable retractors and a light bar Optional portable accessary	Wheeled cart

Mechanical Parameters

Dimensions (W*D*H)	610mm*270mm*610mm Weight (cabinet+module+AC box)	62.5kg+22.5kg+10kg=95kg

*The product is continuously evolving. Please refer to the latest datasheet for specifications.

Introduction

EXP60K3 DC charger is specifically designed for wall-mount and pedestal installations, and is fully compatible with CCS/NACS/ CHAdeMO/GBT EV charging standards. It features a small size and light weight, making it easy to install and relocate, along with a user-friendly interface, IP55 protection, and a robust, durable design for various outdoor applications. It can deliver up to 60kW

charging power and 150Vdc to 1000Vdc charging voltage. It can be configured with optional 22kW/32A AC Type 2 charging cables.

Main Features

Flexible installation options: wall-mount/pedestal

installation

CCS1/CCS2/GBT/CHAdeMO/NACS support, AC Type 2 charging cable support

WEB/APP Maintenance Backend System enhances the operation and maintenance effectiveness

TUV CE, cTUVus, RED, CB, OCA, FCC, CTEP and

Energy Star certification

Optional cable retractor/credit card payment

(POS), VIN code support

Three vehicles charging at the same time

IP55 protection, reliable & robust	     High power density with best space efficiency

BABA support

Operating temperature	-30°C~+70°C, full power output up to 50°C, power derating 5%/°C above 50°C

Transportation/Storage

temperature	-40°C~+75°C

Relative humidity	≤95%RH, non-condensing

Altitude	2000m

Atmospheric pressure	80kPa~110kPa

Cooling method	Air cooling

Protection rating	IP55/IK10 (IK08 for touch screen)

Input Parameters

AC frequency	45Hz~65Hz

AC voltage	CE: 3-phase+N+PE, 380/400Vac±10%; UL: 3-phase+PE, 480Vac±10%

Max input current	CE: 97A+32A (for AC output); UL: 77A

Power factor	>0.99

THDi	<3% (rated input)

Output Parameters

Max DC output power	60kW

DC output voltage	150Vdc~1000Vdc Constant power voltage range	300Vdc~1000Vdc Max DC output current	200A

Optional AC charging output	AC charging cable 32A/3P/22kW

Performance

Max system efficiency	>95% (rated input and output)

Standby power	CE: 44W; UL: 53.2W

Noise	≤72dB

Standard

Backend	OCPP 1.6J, support firmware update to 2.0x, OTA/PnC/smart charging/power management support

CCS PLC communication	DIN70121 and ISOI5118

CHAdeMO	CHAdeMO V1.2

GBT	GB/T 18487.1-2015, GB/T 27930-2015, GB/T 20234.1~3-2015 TUV CE, cTUVus, RED, CB, OCA, FCC, CTEP, Energy Star

Certification

UL 2202/CSA C22.2; EN IEC61851-21-2 Class B; EN IEC61851-1/EN IEC61851-23/EN 61851-24

Manufacture	BABA support

Function and Interface

HMI	Tempered glass protective 7" TFT touch screen LCD, RGB LED, EPO

Metering	DC energy meter, DL/T645 standard, optional MID-certified meter

Monitoring interface	LAN and LTE wireless modem support

Charging mode	Auto/By Time/By Plan/By kWh/By SOC

Power switching mode	Two DC charging connectors charging in sequence, AC Type 2 cable charging in parallel Payment	RFID, Mobile APP, POS (optional)

MBE function support	WEB/APP Maintenance Backend System enhances the operation and maintenance effectiveness

Charging Protection

Overcurrent protection, short circuit protection, overvoltage protection, undervoltage protection

Insulation monitoring, protective earth connection monitoring Reverse polarity protection, overtemperature protection RCDB for AC charging cable protection

Configuration

Charging module	2 * REG1K0100G2 30kW 1000V power module

Charging connector	One or two CCS/NACS/GBT connectors, CHAdeMO + CCS/NACS/GBT connectors, optional AC Type 2 charging cable

Charging connector capacity	GBT: 200A; CCS: 200A; NACS: 200A; CHAdeMO: 125A; AC: 32A/22kW

Optional pedestal	Pedestal with two cable retractors and a light bar

Mechanical Parameters

Dimensions (W*D*H)	705mm*240mm*1100mm Weight (cabinet+module)	92kg+2*15.5kg=123kg

*The product is continuously evolving. Please refer to the latest datasheet for specifications.

EXP180K2-FD (T)/EXP240K5-FD Fast DC Charger

180kW/240kW DC Charging Power with Two DC Charging Connectors + Optional 22kW AC Type 2 Charging Socket

Introduction

EXP180K2-FD (T)/EXP240K5-FD DC charger adopts all-in-one design and is fully compatible with CCS2/NACS/CHAdeMO/ GBT EV charging standards. It features a small size and light weight, making it easy to install and relocate, along with a user- friendly interface, IP55 protection, and a robust, durable structure suitable for various outdoor applications. It can deliver up to 180kW/240kW charging power and 150Vdc to 1000Vdc charging voltage. It can be configured with an optional 22kW/32A AC Type 2 charging socket and Eichrecht certification.

Main Features

TUV CE, cTUVus, RED, CB, OCA, FCC, CTEP,

Energy Star and NMI Eichrecht1 certification

CCS1/CCS2/GBT/CHAdeMO/NACS support, AC Type 2 charging socket support1

Intelligent power switching matrix dynamically allocates power modules among connectors

WEB/APP Maintenance Backend System enhances the operation and maintenance effectiveness

Optional cable retractor, credit card payment

(POS), VIN code support

Ring network power sharing between two chargers1

IP55 protection, reliable & robust	Three vehicles charging at the same time1

Note 1: This feature is supported only by EXP180K2; it is not available on EXP240K5.

Ring Network with Two 180kW Chargers

Each Charging Connector Can Get Full 360kW Power with Ring Network

| Charger A-DC1 | ChargerA-DC2 | Charger B-DC1 | Charger B-DC2 |
| --- | --- | --- | --- |
| 90kW | 90kW | 90kW | 90kW |
| 180kW | 0 | 180kW | 0 |
| 180kW | 90kW | 90kW | 0 |
| 180kW | 180kW | 0 | 0 |
| 360kW | 0 | 0 | 0 |

| Input Parameters | AC input current | 291A | 291A+32A | 388A |
| --- | --- | --- | --- | --- |
|  | Power factor | >0.99 |  |  |
|  | THDi | <3% (rated input) |  |  |
|  | DC output voltage | 150Vdc~1000Vdc |  |  |
|  | Constant power voltage range | 300Vdc~1000Vdc |  |  |

Output Parameters

Performance

Standard

Max DC output current	600A@300V/360A@500V/180A@1000V	600A@400V/480A@500V/240A

Model

EXP180K2-FD

EXP180K2-FT

EXP240K5-FD

Operating temperature	-30°C~+70°C, full power output up to 50°C, power derating 5%/°C above 50°C

Transportation/Storage

temperature	-40°C~+75°C

Operating Environment  Relative humidity	≤95%RH, non-condensing

Altitude	2000m

Atmospheric pressure	80kPa~110kPa

Cooling method	Air cooling

AC frequency	45Hz~65Hz

AC voltage	CE: 3-phase+(N)+PE, 380/400Vac±10%; UL: 480Vac±10%

@1000V

Max DC output power	180kW	180kW+22kW	240kW

AC output	AC Type 2

32A/22kW/3P

Max system efficiency	>95% (rated input and output)	>95.5% (rated input and output)

Standby power	40W

Noise	<65dB

Backend	OCPP 1.6J, support firmware update to 2.0x, OTA/PnC/smart charging/power management support

CCS PLC communication	DIN70121 and ISOI5118 CHAdeMO	CHAdeMO V1.2

GBT	GB/T 18487.1-2015, GB/T 27930-2015, GB/T 20234.1~3-2015

TUV CE, cTUVus, RED, CB, OCA, FCC, CTEP, Energy Star, NMI Eichrecht (for

Certification

EXP180K2-FD)

UL 2202, EN 61851-21-2 Class A; EN 61851-1/EN 61851-23/EN 61851-24

Protection rating	IP55/IK10 (IK08 for touch screen)

HMI	Tempered glass protective 7" TFT touch screen LCD, 3 * RGB LED, EPO

Metering	DC energy meter, GB/T 29318 and JJG1149 standard, optional MID-certified/NMI- certified meter

Monitoring interface	LAN and LTE wireless modem support

Function and Interface

Optional ring network power

transfer

Enables power transfer between two chargers, each charging connector can get the

total charging power

Power switching mode	Intelligent power switching matrix dynamically allocates power modules among

charging connectors, maximizing overall power utilization efficiency

Payment	RFID, Mobile APP, POS (optional)

MBE function support	WEB/APP Maintenance Backend System enhances the operation and maintenance effectiveness

Overcurrent protection, short circuit protection, overvoltage protection, undervoltage protection

Insulation monitoring, protective earth connection monitoring

Charging Protection

Reverse polarity protection, overtemperature protection

RCDA main protection and RCDB protection for AC connector

Charging module	6 * REG1K0100G/U 30kW 1000V	6 * REG1K0110U 40kW 1000V

Charging connector	2 * CCS/NACS/GBT or CCS/NACS+GBT/CHAdeMO, optional AC Type 2 charging socket

Configuration

Charging connector capacity	GBT: 250A/400A; CCS1: 200A/300A; CCS2: 200A/250A/300A/375A;

NACS: 200A/300A; CHAdeMO: 125A/200A; AC: 32A/22kW

Option 1: Two top-mounted cable retractors + light bar

Optional cable management

Option 2: Two top-mounted folding-arm cable retractors + light bar

Dimensions (W*D*H)	700mm*750mm*1750mm

Mechanical Parameters

Weight (cabinet+module)	320kg+6*22.5kg=455kg	320kg+6*23.5kg=461kg

*The product is continuously evolving. Please refer to the latest datasheet for specifications.

EXP480K6-FD Fast DC Charger

480kW DC Charging Power with Two DC Charging Connectors

Introduction

EXP480K6-FD fast DC charger is specifically designed with high power, high current, high protection and high reliability, and is fully compatible with CCS1/CCS2/NACS/GBT EV charging standards. Featuring all-in-one design and IP55 protection, it is

suitable for diverse harsh environments. It delivers up to 480kW of charging power, with a maximum output current of 600A and a maximum voltage of 1000Vdc. The completely updated design features light bars, 32" (optional 10'') LCD and a more user- friendly connector management system.

Main Features

CCS1/CCS2/NACS/GBT support, optional two

600A liquid-cooled charging connectors

TUV CE, cTUVus, RED, CB, OCA, FCC, CTEP, UKCA,

Energy Star and NMI Eichrecht certification

WEB/APP Maintenance Backend System enhances the operation and maintenance effectiveness

User-friendly interface with a 32" (optional 10'')     tempered glass TFT capacitive touch screen LCD

and 2 RGB LED bars

IP55, IK10, -30°C~+50°C operating temperature full power charging

OCPP 1.6J, support firmware update to 2.0x, PnC, smart charging and power management support

Intelligent power switching matrix dynamically allocates power modules among connectors

Supports LAN and LTE wireless connectivity, OTA

updates, RFID authorization, POS and Mobile APP payments

Operating Environment

Operating temperature	-30°C~+70°C, full power output up to 50°C, power derating 5%/°C above 50°C

Transportation/Storage

temperature	-40°C~+75°C

Relative humidity	≤95%RH, non-condensing

Altitude	2000m

Atmospheric pressure	79kPa~106kPa

Cooling method	Forced air cooling

Protection rating	NEMA 3R/IP55/IK10

AC Input

Frequency	45Hz~65Hz

Voltage	CE: 3-phase+N+PE, 380/400V±10%; UL: 3-phase+(N)+PE , 480V±10%

Max input current	CE: 776A; UL: 614A

Power factor	>0.99

THDi	<3% (rated input)

DC Output

Voltage	150Vdc~1000Vdc

Constant power voltage range	300Vdc~1000Vdc

Max output current	1200A@400V/960A@500V/480A@1000V

Max output power	480kW

Performance

Max system efficiency	96.5%

Standby power	100W

Noise	≤65dB

Standard

Backend	OCPP 1.6J, support firmware update to 2.0x, OTA updates, PnC/smart charging/power management

support

CCS PLC communication	DIN70121 and ISOI5118

GBT	GB/T 18487.1-2015, GB/T 27930-2015, GB/T 20234.1~3-2015

TUV CE, cTUVus, RED, CB, OCA, FCC, CTEP, UKCA, Energy Star, NMI Eichrecht

Certification

Function and Interface

UL 2231, UL 991, UL 2202, CSA C22.2 No.346:22, EN 61000-6-3/EN 61000-6-1 Class A, EN 61851-1/EN 61851-23/EN 61851-24

HMI	32" (optional 10'') tempered glass TFT touch screen LCD, two light bars, EPO

Metering	DC energy meter, optional MID-certified and NMI-certified meter

Monitoring interface	LAN and LTE wireless modem support

VPF function	Optional VPF configuration, from 40kW to 480kW with VPF power module

Power switching mode	Intelligent power switching matrix dynamically allocates power modules among charging connectors (with 120kW power step), maximizing overall power utilization efficiency

Payment	RFID, Mobile APP, POS (optional)

MBE function support	WEB/APP Maintenance Backend System enhances the operation and maintenance effectiveness

Charging Protection

Overcurrent protection, short circuit protection, overvoltage protection, undervoltage protection

Insulation monitoring, protective earth connection monitoring Reverse polarity protection, overtemperature protection

Configuration

Charging module	12 * REG1K0135P2 module for VPF function (optional REG1K0135G2 module) Charging connector configuration     2 * CCS2/CCS1/NACS/GBT

Charging connector capacity	GBT: 600A/400A, CCS1: 600A/350A, CCS2: 500A/375A, NACS: 600A/360A

Cable management	Two top-mounted cable rocker arms

Mechanical Parameters

Dimensions (W*D*H)	870mm*750mm*2256mm Weight (cabinet+module)	546kg+12*17kg=750kg

*The product is continuously evolving. Please refer to the latest datasheet for specifications.

EXP300H9-FS MCS Charger

300kW DC Charging Power with One MCS Charging Connector

Introduction

EXP300H9-FS Megawatt Charging System (MCS) charger adopts all-in-one design and is compatible with MCS EV charging standard. It features a small size and light weight, making it easy to install and relocate, along with a user-friendly interface, IP55 protection, and a robust, durable structure suitable for various outdoor applications. It can deliver up to 300kW charging power and 250Vdc to 1500Vdc charging voltage.

Main Features

WEB/APP Maintenance Backend System enhances

the operation and maintenance effectiveness	Credit card payment (POS), VIN code support

Supports MCS charging with voltage up to 1500V	IP55 protection, reliable & robust

Operating temperature	-30°C~+70°C, full power output up to 50°C, power derating 5%/°C above 50°C

Transportation/Storage

temperature	-40°C~+75°C

Relative humidity	≤95%RH, non-condensing Operating Environment  Altitude	2000m

Atmospheric pressure	80kPa~110kPa

Cooling method	Air cooling

Protection rating	IP55/IK10 (IK08 for touch screen)

AC frequency	45Hz~65Hz

AC voltage	CE: 3-phase+(N)+PE, 380/400Vac±10%; UL: 480Vac±10%

Input Parameters

AC input current	CE: 485A; UL: 384A

Power factor	>0.99

THDi	<3% (rated input)

DC output voltage	250Vdc~1500Vdc Constant power voltage range	400Vdc~1500Vdc

Output Parameters

Max DC output current	350A@857V, 300A@1000V, 200A@1500V

Max DC output power	300kW

Max system efficiency	>95% (rated input and output)

Performance

Standard

Standby power	40W

Noise	<65dB

Backend	OCPP 1.6J, support firmware update to 2.0x, OTA/PnC/smart charging/power management support

MCS PLC communication	DIN70121, ISOI5118

HMI	Tempered glass protective 7" TFT touch screen LCD, RGB LED, EPO

Metering	DC energy meter

Monitoring interface	LAN and LTE wireless modem support

Function and Interface

Payment	RFID, Mobile APP, POS (optional)

MBE function support	WEB/APP Maintenance Backend System enhances the operation and maintenance effectiveness

Overcurrent protection, short circuit protection, overvoltage protection, undervoltage protection

Insulation monitoring, protective earth connection monitoring

Charging Protection

Reverse polarity protection, overtemperature protection

RCDA main protection (CE version only)

Charging module	6 * REG1K5125G2 50kW 1500V

Configuration

Charging connector	1 * MCS

Charging connector capacity	MCS: 350A

Dimensions (W*D*H)	700mm*750mm*1750mm

Mechanical Parameters

Weight (cabinet+module)	320kg+6*20kg=440kg

*The product is continuously evolving. Please refer to the latest datasheet for specifications.

600kW Power Cube/12 Charging Connectors/Overhead Charging Dispenser Solution

Introduction

EXP600K3-FWR5 split type high power charger is specifically designed for EV charger operators with flexible split and multi charging point solutions. It supports CCS/NACS/CHAdeMO/GBT charging connectors, delivers up to 600kW of output power, and accommodates up to 12 charging connectors via 6 dual-connector dispensers or 12 single-connector dispensers. It also supports 500A liquid-cooled charging connectors, and each connector can receive maximum power through intelligent power switching. An intelligent power switching matrix dynamically allocates power modules among charging connectors, maximizing overall power utilization efficiency. The overhead dispenser solution for bus depots is also available, utilizing the same power cube.

Main Features

Building-block modular design, simplified & standardized interface

Max 12 charging channels, 300A or 500A

configuration

TUV CE, cTUVus, RED, CB, OCA, FCC, CTEP and

Energy Star certification

Power cube IP55, floor-standing dispenser IP55,

overhead dispenser IP65, reliable & robust Patented U-Type air duct design

Customizable dispenser, overhead dispenser solution for bus depots

WEB/APP Maintenance Backend System enhances the operation and maintenance effectiveness

High power density with best space efficiency Power cube 65dB, floor-standing dispenser 55dB,

overhead dispenser 0dB

BABA support

Power Cube	EXP600K3-FWR5

Frequency	45Hz-65Hz

Voltage	CE: 3-phase+(N)+PE, 380Vac/400Vac±10%;  UL: 3-phase+(N)+PE, 480Vac±10%

AC Input

DC Output

Performance

Max input current	CE: 2*480A@380Vac; UL: 2*380A@480Vac Power factor	>0.99

THDi	<5% (rated input)

Voltage	150Vdc~1000Vdc

Constant power voltage

range	300Vdc~1000Vdc

Max output current	500A (optional 300A) for each output charging channel

Max output power	600kW

Output channel	Max 12 output charging channels as the input of the charging dispensers Dispenser support	Max 6 dual-connector dispensers or 12 single-connector dispensers System efficiency	>95.5% (rated input and output)

Standby power	70W

Noise	<65dB

Dimensions (W*D*H)	1050mm*1150mm*2200mm

Mechanical Parameters

Weight (cabinet+module)    690kg+20*15.5kg=1000kg

Power module	20 * REG1K0100G2 (30kW 1000V)

Power switching mode	Intelligent power switching matrix dynamically allocates power modules among charging connectors, maximizing overall power utilization efficiency

HMI	RGB LED, E-STOP

Others	 		 HMI kiosk (optional)	Tempered glass protective 10.1" capacitive touch screen LCD, RFID, POS (optional)

Protection rating	IP55

Certification	TUV CE, cTUVus, RED, CB, OCA, FCC, CTEP, Energy Star

Manufacture	BABA support

Floor-Standing Dispenser	Overhead Dispenser

Charging Dispenser

|  | Input channel (DC bus) | 2 |  |  |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Charging cable | 2 |  |  |  | 1 |  |
|  | Voltage | 150Vdc~1000Vdc |  |  |  |  |  |
| DC Output | Max output current | 2*250A | 2*375A | 500A+250A | 2*500A | 250A | 375A |
|  | Optional (boost current) | 2*375A | 2*500A | 700A+375A | 2*700A | 375A | 500A |
|  | Max output power | 2*250kW | 2*375kW | 500kW+250kW | 2*500kW | 250kW | 375kW |

DC Input

EXP250-FDC	EXP400-FDC	EXP500-LDC	EXP500-LLDC	EXP250-FSW	EXP400-FSW

Voltage	150Vdc~1000Vdc

Dimensions (W*D*H)	500mm*260mm*1800mm	763mm*240mm*415mm

Mechanical Parameters

Weight	185kg	195kg	50kg

HMI	Tempered glass protective 7" capacitive touch screen LCD for floor-standing dispenser, RGB panel LED, E-STOP,

RFID, POS (optional)

Metering	DC energy meter, GB/T 29318 and JJG1149 standard, optional MID-certified meter

Backend	OCPP 1.6J, support update to 2.0, PnC/smart charging/power management support, LAN 100M and LTE wireless modem support

Cable management	Top-mounted two cable retractors and a light bar for floor-standing dispenser. Select one or two cable

retractors based on cable length for overhead dispenser

MBE function	WEB/APP Maintenance Backend System enhances the operation and maintenance effectiveness

Others

Overcurrent/short circuit/overvoltage/undervoltage/RCD/insulation/overtemperature protection

Charging protection	 	 Reverse polarity protection

Charging connector	2*GBT/CCS/NACS or CCS+NACS/GBT/CHAdeMO or 1*GBT/CCS/NACS      CCS/GBT/NACS

Charging connector capacity

CCS: 200A/250A/300A/375A/500A; NACS:200A/300A/380A; CHAdeMO: 125A/200A;

GBT: 250A/400A/500A

Protection rating	IP55/IK10	IP65

Certification	TUV CE, cTUVus, RED, CB, OCA, FCC, CTEP, Energy Star

Noise	0	55dB	0

Operating Environment

Operating temperature

Transportation/Storage

temperature Relative humidity Altitude

Atmospheric pressure

Cabinet cooling

-30°C~+70°C, full power output up to 50°C, power derating 5%/°C above 50°C

-40°C~+75°C

≤95%RH, non-condensing 2000m

80kPa~110kPa Air cooling

Standard

Charging standard

EMC/safety

GBT 27930-2011/27930-2015, CHAdeMO V1.2/V2.0, DIN70121, ISO15118, OCA

EN 61851-21-2, EN 61000-6-4, EN 61000-6-2 Class A, EN 61851-1, EN 61851-23, EN 61851-24, UL 2202, CSA C22.2

*The product is continuously evolving. Please refer to the latest datasheet for specifications.

800kW Power Cube/12 Charging Connectors/MCS Dispenser/Overhead Charging Dispenser Solution

Introduction

EXP800K6-FWR8 split type high power charger is specifically designed for the EV charger operators with split and multi charging point applications. It supports MCS/CCS/NACS/GBT charging connectors, delivers up to 800kW/1.6MW of output power, and accommodates up to 12 charging connectors via 6 dual-connector dispensers or 12 single-connector dispensers.

It also supports 500A/600A/1500A liquid-cooled charging connectors, and each connector can receive maximum power through intelligent power switching. An intelligent power switching matrix dynamically allocates power modules among charging connectors, maximizing overall power utilization efficiency. Two power cubes can be connected in parallel to achieve 1.6MW output, equipped with an MCS charging dispenser.

Main Features

Building-block modular design, simplified & standardized interface

Up to 12 charging channels, configurable for 300A, 500A, or max 800A output

TUV CE, cTUVus, RED, CB, FCC, OCA, and NMI

Eichrecht certification

Power cube IP55, floor-standing dispenser IP55,

overhead dispenser IP65, reliable & robust

High power density with best space efficiency

BABA support

Customizable dispenser, overhead dispenser solution for bus depots

WEB/APP Maintenance Backend System enhances the operation and maintenance effectiveness

Supports two power cubes in parallel for one

dispenser, delivering up to 1.6MW output power

Power cube 65dB, floor-standing dispenser 55dB,

overhead dispenser 0dB

Patented U-Type air duct design

Power Cube	EXP800K6-FWR8

Frequency/Voltage	45Hz~65Hz; CE: 3-phase+(N)+PE, 380/400Vac±10%; UL: 3-phase+(N)+PE, 480Vac±10%

Max input current	CE: 2*640A@380Vac; UL: 2*506A@480Vac

AC Input

Power factor	>0.99

THDi	<5% (rated input)

Voltage	150Vdc~1000Vdc (constant power voltage range 300Vdc~1000Vdc) Max output current	800A for each output charging channel

DC Output

Performance

Max output power	800kW

Output channel	Max 12 output charging channels as the input of the charging dispensers Dispenser support	Max 6 dual-connector dispensers or 12 single-connector dispensers System efficiency	97% (peak efficiency)

Standby power	70W

Noise	<65dB

Dimensions (W*D*H)	1050mm*1100mm*2200mm

Mechanical Parameters

Weight (cabinet+module)    800kg+20*17kg=1140kg

Power module	20 * REG1K0135G2 (40kW 1000V)

Power switching mode	Intelligent power switching matrix dynamically allocates power modules among charging connectors, maximizing overall power utilization efficiency

HMI	RGB LED, E-STOP

Others	 		 HMI kiosk (optional)	Tempered glass protective 10.1" capacitive touch screen LCD, RFID, POS (optional)

Protection rating	IP55

Certification	TUV CE, cTUVus, RED, CB, OCA

Manufacture	BABA support

Floor-Standing Dispenser

Overhead Dispenser G3 Dispenser	MCS Dispenser

Charging Dispenser

EXP250-FFC      EXP400-FFC     EXP500-LL5C

EXP600-LL6C

EXP800-LC	EXP1500-HLL5C/6C	EXP250/400-FSW

DC Input	 					 Input channel	2	1	2/3	1

Charging cable	2	1	2	1

Voltage	150Vdc~1000Vdc

DC Output

Max output current	2*250A	2*375A	2*500/600A	800A	1500A+500A/600A	250A/375A Optional (boost current)      2*375A	2*500A	2*700A	1000A	1500A+700A	375A/500A Max output power	2*250kW	2*375kW	2*500/600kW   800kW	800kW/1600kW	250kW/375kW

Dimensions (W*D*H)	560*300*2000mm	355*730*2200mm	763*240*415mm

Mechanical Parameters

Weight	230kg	260kg	330kg	50kg

HMI	Tempered glass protective 10.1'' (optional 21.5'') capacitive touch screen LCD, RGB panel LED, E-STOP, RFID, QR

code, POS (optional)

Metering	DC energy meter, MID-certified meter, optional NMI-certified meter

Backend	OCPP 1.6J/2.0.1 (optional 2.3X), PnC/OTA/smart charging/power management support, LTE wireless modem/

Ethernet

Cable management	Top-mounted swing-arm for cable retraction

MBE function	WEB/APP Maintenance Backend System enhances the operation and maintenance effectiveness Overcurrent/short circuit/overvoltage/undervoltage/RCD/insulation/overtemperature protection

Charging protection

Reverse polarity protection

Others

CCS+MCS/NACS+MCS/

Charging connector	2*CCS/NACS/GBT or CCS+NACS/GBT or 1*CCS/NACS/GBT

Charging connector

GBT+MCS/CCS+CCS, or  CCS/GBT/NACS

1*MCS

capacity	MCS: 1500A; CCS: 200A/250A/300A/375A/500A/600A; NACS: 200A/300A/380A/600A; GBT: 250A/500A/600A

Protection rating	IP55 (NEMA 3S)/IK10 (IK08 for 10.1" LCD, IK09 for 21.5" LCD)	IP55 (NEMA 3S)/IK10

(IK08 for 10.1" LCD)

IP65

Certification	TUV CE, cTUVus, RED, CB, FCC, OCA, NMI Eichrecht

Noise	0	55dB	0

Operating Environment

| Operating temperature Transportation/Storage temperature Relative humidity Altitude Atmospheric pressure | -30°C~+70°C, full power output up to 50°C, power derating 5%/°C above 50°C -40°C~+75°C ≤95%RH, non-condensing 3000m 80kPa~110kPa |
| --- | --- |
| Cabinet cooling | Air cooling |

Standard

Charging standard

EMC/safety

GBT 27930-2011/27930-2015, DIN70121, ISO15118, OCA

EN 61851-21-2, EN 61000-6-4, EN 61000-6-2 Class A, EN 61851-1, EN 61851-23, EN 61851-24, UL 2202, CSA C22.2

*The product is continuously evolving. Please refer to the latest datasheet for specifications.

IHEBS470K7D-350LL3C Liquid-Cooled Split Type Battery Buffer Charger

400kW Charging Power/6 Charging Connectors/350kWh Charger

Introduction

IHEBS470K7D-350LL3C Liquid-Cooled Split Type Battery Buffer Charger utilizes LFP battery for energy storage. With the support of local and cloud-based EMS, it helps balance power supply and demand among the grid, battery system, EV charging and renewable energy sources. A fully liquid-cooled solution enables 1P discharge performance, allowing the system to operate under limited AC grid capacity while still delivering high-power charging. This approach achieves high cost efficiency and a compact footprint. The system provides significant value in grid peak shaving, renewable energy integration, and grid capacity expansion by discharging battery power to grid.

Main Features

Fully liquid-cooled design from power modules to battery packs and charging cables

Total electrical isolation between the grid,

battery, EV and PV input

1P fast charging/discharging

2 option 1

Intelli

gent Power Switch

Matrix

Liquid-cooled units

MPPT DCDC

ACDC   DCDC

PV

AC Grid

PV

AC Bus

Battery alarm

light and beeper Controller unit

EMS controller

MPPT

DC2DC

MPPT

DC2DC

DC/DC

AC/DC

Bidi

AC2DC

AC/DC

DC/DC

2 option 1

LCD

DC2DC

EPO

DC/DC

DC/DC	DC/DC

DC/DC	DC/DC

DC/DC	DC/DC

Group 1

Group 2

Group 3

Group 4

DC/DC	DC/DC	DC/DC

Group 5	Group 6

Power switching

matrix unit box

Battery

control box

Energy & Power

Cube

#1 Charging

Dispenser

#2 Charging

Dispenser

#3 Charging

Dispenser

Ultra-low noise

Battery pack

Highly integrated solution combining PV, BESS, EV charging and V2G functions

Micro-grid enabling high-power charging       Easy relocation between sites

Backup power enabled by B2G

Power Cube	IHEBS470K7D-350LL3C 470kW/350kWh

Frequency/Voltage	50Hz/60Hz, 3-phase+N+PE, 380/400Vac±10%

AC Input (On-Grid)

Max input current/power    133A/70kW

Power factor	0.8 leading ~ 0.8 lagging

Voltage	DC+/- +PE, 300Vdc~740Vdc, max 900Vdc

PV Input	 	 Max input current/power    70A (with 1 PV module); 140A (with 2 PV modules)/40kW (with 1 PV module); 80kW (with 2 PV modules)

Voltage	150Vdc~1000Vdc

Max output current	6 outputs, each output max 600A

DC Output	 		 Max output power	400kW (320kW @300Vdc~1000Vdc)

Output channel	Max 6 output charging channels via 3 dual-connector dispensers or 6 single-connector dispensers LCG1K0135G-M	DC2DC converter, 40kW/1kV, MPPT support      Max 2 * 40kWp PV input support (optional)

LCG1K0135G	DC2DC converter, 40kW/1kV	Max 10 * 40kW DC bus discharging power/EV charging power

Power Module/ 12 Slots        			 LRG1K0100G	AC2DC rectifier, 30kW/1kV	Max 2 * 30kW DC bus charging power

LBG1K0120G	Bidirectional AC2DC inverter, 35kW/1kV	Max 2 * 35kW AC feedback power (optional)

Energy storage battery	2*175kWh/285Ah liquid-cooled battery packs, 547.2~696.96Vdc, 1P dis/charge, max 1.1P discharge for 10 min

Battery Pack

Performance

High voltage control box     Two Infypower high voltage control boxes

BMS	Infypower 2-level BMS structure: battery pack BSU + high voltage control box BMU

System efficiency	94%

Standby power	216W

Noise	<60dB

Dimensions (W*D*H)	2800mm*1400mm*2200mm

Mechanical Parameters

Weight (cabinet+module)   6140kg+12*30kg=6500kg

Power switching mode	Intelligent power switching matrix dynamically allocates power modules among charging connectors, maximizing overall power utilization efficiency

HMI	10" TFT touch screen, 2 RGB LED, E-STOP, battery alarm light and beeper 1 AC energy meter, optional MID-certified meter

Metering

1 or 2 DC energy meters, optional NMI Eichrecht-certified meter

Others

Charging Dispenser

|  | Input channel (DC bus) | 2 |  |  |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Charging cable | 2 |  |  |  | 1 |  |
|  | Voltage | 150Vdc~1000Vdc |  |  |  |  |  |
| DC Output | Max output current | 2*250A | 2*375A | 500A+250A | 2*500A | 250A | 375A |
|  | Optional (boost current) | 2*375A | 2*500A | 700A+375A | 2*700A | 375A | 500A |
|  | Max output power | 2*250kW | 2*375kW | 500kW+250kW | 2*500kW | 250kW | 375kW |

DC Input

Fire suppression system      Completely submerged aerosol fire extinguishing system, water fire interface, emergency fan Safety design	Explosion-proof board configuration and fire-proof spread structure design

Environment sensor	Water/temperature/humidity/door access/smoke/combustible gas sensor Protection rating	NEMA 3R, IP54 for power cube, IP55 for battery cabinet, IP67 for battery pack Certification	TUV CE, CB

Grid connection	EN 50549, VDE-AR-N 4105, VDE-AR-N 4110

Floor-Standing Dispenser	Overhead Dispenser

EXP250-FDC	EXP400-FDC	EXP500-LDC	EXP500-LLDC	EXP250-FSW	EXP400-FSW

Voltage	150Vdc~1000Vdc

Dimensions (W*D*H)	500mm*260mm*1800mm	763mm*240mm*415mm

Mechanical Parameters

Weight	185kg	195kg	50kg

HMI	Tempered glass protective 7" capacitive touch screen LCD for floor-standing dispenser, RGB panel LED, E-STOP,

RFID, POS (optional)

Metering	DC energy meter, GB/T 29318 and JJG1149 standard, optional MID-certified meter

Monitoring interface	OCPP 1.6J, support update to 2.0, PnC/smart charging/power management support, LAN 100M and LTE wireless modem support

Cable management	Top-mounted two cable retractors and a light bar for floor-standing dispenser. Select one or two cable

retractors based on cable length for overhead dispenser

MBE function	WEB/APP Maintenance Backend System enhances the operation and maintenance effectiveness

Others

Overcurrent/short circuit/overvoltage/undervoltage/RCD/insulation/overtemperature protection

Charging protection	 	 Reverse polarity protection

Charging connector	2*GBT/CCS/NACS or CCS+NACS/GBT/CHAdeMO or 1*GBT/CCS/NACS      CCS/GBT/NACS

Charging connector capacity

CCS: 200A/250A/300A/375A/500A; NACS: 200A/300A/380A; CHAdeMO: 125A/200A;

GBT: 250A/400A/500A

Protection rating	IP55/IK10	IP65

Certification	TUV CE, cTUVus, RED, CB, OCA, FCC, CTEP, Energy Star

Noise	0	55dB	0

Operating Environment

Operating temperature

Transportation/Storage

temperature Relative humidity Altitude

Atmospheric pressure

Cabinet cooling

-20°C~+70°C, full power output up to 50°C, power derating 5%/°C above 50°C

-30°C~+60°C

≤95%RH, non-condensing 2000m

80kPa~110kPa

Liquid cooling

*The product is continuously evolving. Please refer to the latest datasheet for specifications.

G3 Floor-Standing Charging Dispenser

200A/250A/300A/375A/380A/500A/600A Charging Connector

Introduction

The G3 floor-standing charging dispenser is specifically designed for EV charging operators, offering flexible split type system configurations and support for multiple charging points. It is compatible with CCS1/CCS2/NACS/GBT standards and can be configured with air-cooled charging connectors rated up to 200A (CCS1, CCS2, NACS), 250A (CCS2, GBT), 300A (CCS1, CCS2, NACS), 375A (CCS2), and 380A (NACS), as well as liquid-cooled charging connectors rated up to 500A (CCS2, GBT) and 600A (CCS1, NACS, GBT). The corresponding maximum output power for these configurations is 200kW, 250kW, 300kW, 375kW, 380kW, 500kW, and 600kW, respectively.

Main Features

IP55 (NEMA 3S)/IK10 protection rating, full-    power charging from -30°C to +50°C

Optional 600A liquid-cooled charging connector

with Any Cooling™ technology

Full safety functions: output contactor and fuse,

ESD, leakage switch, insulation detector, and

multiple-layer software protection logic

Starts charging with a simple plug-and-charge process after authentication

LTE wireless modem/Ethernet, RFID, POS and mobile app payment options

User-friendly interface with a 10.1'' (optional

21.5'') TFT capacitive touch screen LCD protected by tempered glass

Flexible swing-arm cable retraction management for the charging connector

Model	EXP250-FFC	EXP400-FFC	EXP500-LL5C EXP600-LL6C

EXP800-LC

Operating

Environment

Operating temperature   -30°C~+70°C, full power output up to 50°C, power derating 5%/°C above 50°C Transportation/Storage

temperature	-40°C~+75°C

Relative humidity	≤95%RH, non-condensing

Altitude	2000m

Atmospheric pressure	80kPa~110kPa Cabinet cooling	Air cooling

Cable cooling	Natural cooling	Liquid cooling Protection rating	IP55 (NEMA 3S)/IK10 (IK08 for 10.1" LCD, IK09 for 21.5" LCD) Noise	0	55dB

Voltage	150Vdc~1000Vdc

DC Input	 		 Input channel	2

|  | Charging cable | 2 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Voltage | 150Vdc~1000Vdc |  |  |  |
| DC Output | Max output current | 2*250A | 2*375A | 2*500A/600A | 800A |
|  | Optional (boost current) | 2*375A | 2*500A | 2*700A | 1000A |
|  | Max output power | 2*250kW | 2*375kW | 2*500kW/600kW | 800kW |

Standard

Certification

Charging standard

EMC/safety

HMI

Metering

Function and

Interface

Backend

Cable management MBE function

Charging protection

Configuration

Mechanical

Parameters

Charging connector

Charging connector capacity

Dimensions (W*D*H)

Weight

TUV CE, cTUVus, RED, CB, FCC, OCA, NMI Eichrecht

NB/T 33001, NB/T 33008, EN 61851-23/24, DIN70121, ISO15118 (optional ISO15118-20) EN 61851-21-2, EN 61000-6-2/4 Class A, EN 61851-1/23/24, UL 2202, CSA C22.2

Tempered glass protective 10.1'' (optional 21.5'') capacitive touch screen LCD, RGB panel LED, E-STOP, RFID, QR code, POS (optional)

DC energy meter, MID-certified meter, optional NMI-certified meter

OCPP 1.6J/2.0.1 (optional 2.3X), PnC/OTA/smart charging/power management support, LTE wireless modem/Ethernet

Top-mounted swing-arm for cable retraction

WEB/APP Maintenance Backend System enhances the operation and maintenance effectiveness

Overcurrent/short circuit/overvoltage/undervoltage/RCD/insulation/overtemperature

protection

Reverse polarity protection

2*CCS/NACS/GBT or CCS+NACS/GBT or 1*CCS/NACS/GBT

CCS: 200A/250A/300A/375A/500A/600A; NACS: 200A/300A/380A/600A;

GBT: 250A/500A/600A

560mm*300mm*2000mm

230kg

260kg

*The product is continuously evolving. Please refer to the latest datasheet for specifications.

EXP1500 MCS Charging Dispenser

200A/250A/300A/375A/380A/500A/600A/1500A Charging Connector

Introduction

EXP1500 Megawatt Charging System (MCS) charging dispenser is specifically designed for EV charging operators, offering flexible split type system configurations and support for multiple charging points. It is compatible with CCS1/CCS2/NACS/GBT/ MCS standards and can be configured with air-cooled charging connectors rated up to 200A (CCS1, CCS2, NACS), 250A (CCS2, GBT), 300A (CCS1, CCS2, NACS), 375A (CCS2), and 380A (NACS), as well as liquid-cooled charging connectors rated up to 500A (CCS2, GBT), 600A (CCS1, NACS, GBT) and 1500A (MCS). The corresponding maximum output power for these configurations is 200kW, 250kW, 300kW, 375kW, 380kW, 500kW, 600kW, and 1500kW, respectively.

Main Features

IP55 (NEMA 3S)/IK10 protection rating, full-    power charging from -30°C to +50°C

Optional 1500A liquid-cooled charging

connectors with Any Cooling™ technology

Flexible swing-arm cable retraction management for the charging connector

Full safety functions: output contactor and fuse,

ESD, leakage switch, insulation detector, and

multiple-layer software protection logic

Starts charging with a simple plug-and-charge process after authentication

LTE wireless modem/Ethernet, QR code and POS payment options

User-friendly interface with a 10.1'' TFT capacitive touch screen LCD protected by tempered glass

| Model |  | EXP1500-HLL5C (CE) EXP1500-HLL6C (UL) -30°C~+70°C, full power output up to 50°C, power derating 5%/°C above 50°C |
| --- | --- | --- |
| Operating Environment | Operating temperature |  |
|  | Transportation/Storage temperature | -40°C~+75°C |
|  | Relative humidity | ≤95%RH, non-condensing |
|  | Altitude | 2000m |
|  | Atmospheric pressure | 80kPa~110kPa |
|  | Cabinet cooling | Liquid cooling |
|  | Cable cooling | Air cooling/liquid cooling |
|  | Protection rating | IP55 (NEMA 3S)/IK10 (IK08 for 10.1" LCD) |
|  | Noise | 55dB |
| DC Input | Voltage | 150Vdc~1500Vdc |
|  | Input channel | 2 or 3 (2 for MCS 1500A + optional 1 for CCS 600A) |
| DC Output | Charging cable | 2 |
|  | Voltage | 150Vdc~1500Vdc |
|  | Max output current | 1500A + 600A |
|  | Optional (boost current) | 1500A + 700A |
|  | Max output power | 1500kW + 600kW |
| Standard Function and Interface | Certification | TUV CE, cTUVus, RED, CB, FCC, OCA, NMI Eichrecht |
|  | Charging standard | DIN70121, ISO15118 (optional ISO15118-20) |
|  | EMC/safety HMI | EN 61851-21-2, EN 61000-6-2/4 Class A, EN 61851-1/23/24, EN 61851-23-3, UL 2202, CSA C22.2 Tempered glass protective 10.1'' capacitive touch screen LCD, RGB panel LED, E-STOP, QR code, POS (optional) |
|  | Metering Backend | DC energy meter, MID-certified meter, optional NMI-certified meter OCPP 1.6J/2.0.1 (optional 2.3X), PnC/OTA/smart charging/power management support, LTE wireless modem/Ethernet |
|  | Cable management | Top-mounted swing-arm for cable retraction |
|  | MBE function Charging protection | WEB/APP Maintenance Backend System enhances the operation and maintenance effectiveness Overcurrent/short circuit/overvoltage/undervoltage/RCD/insulation/overtemperature protection Reverse polarity protection |
| Configuration | Charging connector Charging connector capacity | CCS+MCS/NACS+MCS/GBT+MCS/CCS+CCS, or 1*MCS MCS: 1500A; CCS: 200A/250A/300A/375A/500A/600A; NACS: 200A/300A/380A/600A; GBT: 250A/500A/600A |
| Mechanical Parameters | Dimensions (W*D*H) Weight | 355mm*730mm*2200mm 330kg |

*The product is continuously evolving. Please refer to the latest datasheet for specifications.

IHEBV470K7D-350LL3C Liquid-Cooled All-in-One Movable Battery Buffer Charger

400kW Charging Power/2 Charging Connectors/350kWh Charger

Introduction

IHEBV470K7D-350LL3C Liquid-Cooled Movable All-in-One Battery Buffer Charger supports temporary rescue charging and off- grid power supply services. With the support of local and cloud-based EMS, it helps balance power supply and demand among the grid, battery system, EV charging and renewable energy sources. A fully liquid-cooled battery design enables 1P discharge performance, ensuring compliance with AC grid power limits while delivering high-power charging with improved energy cost efficiency and space utilization, as well as extended battery life. Its fully off-grid electrical design and reinforced structural design make it suitable for long-term transportation under vibration conditions and temporary on-site charging applications.

Main Features

Fully liquid-cooled design from power modules to battery packs and charging cables

Total electrical isolation between the grid,

battery, and EV

1P fast charging/discharging

Ultra-low noise

All-in-one design integrating BESS, EV

charging and V2G functions

Micro-grid enabling high-power charging      Movable application, easy relocation

Backup power enabled by B2G

Operating temperature	-20°C~+70°C, full power output up to 50°C, power derating 5%/°C above 50°C Transportation/Storage

temperature	-30°C~+60°C

Relative humidity	≤95%RH, non-condensing

Operating Environment

Altitude	2000m

Atmospheric pressure	80kPa~110kPa

Cooling method	Liquid cooling

Protection rating	NEMA 3R, IP54 for power cube, IP55 for battery cabinet, IP67 for battery pack Frequency	45Hz~65Hz

Voltage	3-phase+(N)+PE/260Vac-530Vac

Max input power	70kW

AC Input (On-Grid)

DC Output

Max output power

(optional)	70kW

Off-grid operation	Support

Grid access interface	Industrial socket inlet Voltage	150Vdc~1000Vdc

Max output current	375A

Max output power	400kW

Output channel	2

Standby power	246W

Performance

Noise	<60dB

AC input side	1 AC energy meter, optional MID-certified meter

Metering

DC output side	1 or 2 DC energy meters, optional NMI Eichrecht-certified meter

LCG1K0135G	DC2DC converter, 40kW/1kV	10 * 40kW ES battery bus discharging/EV charging power

Power Module/12 Slots	 			 LBG1K0120G	Bidirectional AC2DC inverter, 35kW/1kV	2 * 35kW AC power

Energy storage battery	2*175kWh/285Ah liquid-cooled battery packs, 547.2~696.96Vdc, 1P dis/charge, max 1.1P discharge for 10 min

Battery Pack Mechanical Parameters

Others

High voltage control box     Two Infypower high voltage control boxes

BMS	Infypower 2-level BMS structure: battery pack BSU + high voltage control box BMU Dimensions (W*D*H)	2800mm*1400mm*2200mm

Weight (cabinet+module)    6640kg+12*30kg=7000kg

Charging modes	Auto/By Time/By Plan/By kWh/By SOC

Charging connector	2 * CCS/NACS/GBT, or CCS + NACS/GBT

Power switching mode	Simultaneous charging: both charging connectors can receive 200kW

Sequential charging: both charging connectors can receive 400kW

HMI	Tempered glass protective 10" capacitive touch screen LCD, RGB LED, E-STOP, battery alarm light and beeper Payment	RFID, optional credit card reader installation

Monitoring interface	OCPP 1.6J, support update to 2.0, PnC/smart charging/power management support, LAN 100M and LTE wireless modem support

Certification	TUV CE, CB

Grid connection	EN 50549, VDE-AR-N 4105, VDE-AR-N 4110

*The product is continuously evolving. Please refer to the latest datasheet for specifications.

AC Grid

AC Bus

AC/DC

LBG

BIDI AC2DC

AC/DC

LCG

DC2DC

DC/DC      DC/DC      DC/DC      DC/DC      DC/DC

DC/DC      DC/DC      DC/DC      DC/DC

DC/DC

Energy and

Power Cube

V2G Charging Solution

22kW/44kW/66kW/132kW V2G DC Fast Charger

|  | EXP132K3E-FD |  | EXP66K3E-FD |  | EXP44K3E-FDW |  | EXP22K3E-FSW |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 132kW V2G Charger |  | 66kW V2G Charger |  | 44kW V2G Charger |  | 22kW V2G Charger |

GRID

3*22kW Bidirectional

AC2DC CONVERTER

(1~2)*22kW Bidirectional

AC2DC CONVERTER

GRID

AC2DC CONVERTER

GRID

22kW Bidirectional

AC2DC CONVERTER

22kW Bidirectional AC2DC CONVERTER

GRID

Battery Energy

Storage System

GRID

6*22kW Bidirectional

AC2DC CONVERTER

22kW Bidirectional

22kW Bidirectional

AC2DC CONVERTER

GRID

(3~5)*22kW Bidirectional AC2DC CONVERTER

GRID

Battery Energy

(1~2)*22kW Bidirectional AC2DC CONVERTER

Battery Energy Storage System

22kW Bidirectional AC2DC CONVERTER

(1~3)*22kW Bidirectional AC2DC CONVERTER

Introduction

Storage System

The V2G charger maintains a balanced power supply and demand between the grid and the EV battery by utilizing the EV battery as an energy storage device. Supported by a local or remote Energy Management System (EMS), it enables flexible access to PV energy to optimize grid peak-valley electricity utilization, enhance grid capacity, and provide reliable backup power. It functions as a core node for smart grid or microgrid integration, while also serving as a valuable supplement to user-side energy storage systems.

Main Features

Flexible system configuration, scalable capacity, bidirectional power flow, customization support

Full electrical isolation between the grid, battery

and EV

Support global V2G standards

WEB/APP Maintenance Backend System enhances

the operation and maintenance effectiveness

Compatible with various system configurations Unified EMS strategy

Charging Strategy

Cloud EMS

OCPP 1.6J/2.0

Grid Capacity Enhancement

Backup Power Supply

Energy Management Based on the Status of Grid Side and EV Side

PV Status

EV Charging/Discharging Plan: Time/Grid Power/ES Battery Capacity/ Power Priority

Charging Implementation

ISO15118/-20

DIN70121

26	Field User

Requirement

Peak Valley Price Difference

Grid Quality Optimization

Energy Management Based on the Frame of Calendar and Time

ES Battery Dis/charging Plan: Time/SOC/Alarm/Status/ Electricity Price

V2G Charger	EXP22K3E-FSW	EXP44K3E-FDW	EXP66K3E-FD	EXP132K3E-FD

Operating temperature	-30°C~+70°C, full power output below 50°C, power derating 5%/°C above 50°C

Transportation/Storage

temperature	-40°C~+70°C

Cooling method	Air cooling

Operating Environment

Relative humidity	≤95%RH, non-condensing

Altitude	2000m

Atmospheric pressure	80kPa~110kPa

Place of installation	Indoor/Outdoor

Protection rating	IP55/IK10

AC mode	45Hz~65Hz, 3-phase+(N)+PE, 260Vac~530Vac

Max AC feedback power	22kW	44kW	66kW	132kW

AC Grid Access

Power factor	≥0.99

THDi	<3% (rated input)

Max charging power	22kW	44kW	66kW	132kW

EV Charging

Charging power switching      Single charging connector  Charging in parallel with half power or in sequence with full power between 2

charging connectors

Charging voltage	150Vdc~1000Vdc

Max V2G feedback power      22kW	44kW	66kW	132kW

EV Discharging	 		 V2G protocol standard	CCS: DIN 70121, IEC 15118/-2/-20

System efficiency	≥94.5%

Performance

Metering

Standby power	39W	53.2W	60W

Noise	≤65dB	≤70dB	65dB

AC grid side	One bidirectional AC energy meter

Charging side

One bidirectional DC

energy meter	Two bidirectional DC energy meters

HMI	Tempered glass protective 7" TFT touch screen LCD, RFID, RGB LED, POS (optional)

Back-end platform	OCPP 1.6J, supports firmware update to 2.0x, OTA, PnC/smart charging/power management support

Function and Interface

MBE function support	WEB/APP Maintenance Backend System enhances the operation and maintenance effectiveness

Charging management	Pedestal with two cable retractors and the light bar

Certification	TUV CE, cTUVus, RED

Reliability and Safety

EMC/Safety	UL 2202, EN 62909, EN 61000-6-3/EN 61000-6-1

Class B; EN 61851-1/EN 61851-23/EN 61851-24

Grid connection	VDE-AR-N 4105, EN 50549, UL 1741SB, IEEE 2030.5

UL 2202, EN 62909, EN 61000-6-3/EN 61000-6-1

Class A; EN 61851-1/EN 61851-23/EN 61851-24

BEG1K075G 22kW 1000V	1	2	3	6

Power Module	 		 Charging connector	CCS1/CCS2/NACS

Dimensions (W*D*H)	640mm*270mm*610mm    705mm*240mm*1100mm  700mm*750mm*1750mm

|  | Weight (cabinet+module) | 70kg+14kg=84kg | 90kg+2*14kg=118kg | 205kg+3*14kg=247kg 205kg+6*14kg=289kg |
| --- | --- | --- | --- | --- |
| V2X Box |  | IAU63ATS-AW | IAU125ATS-AW |  |
|  | Frequency | 50Hz/60Hz |  | 60Hz |

Mechanical Parameters

AC Grid Input

Rated voltage	3-phase+N+PE, 380/400Vac±10% (CE), 480Vac±10% (UL)

Current	Max 63A	Max 250A

3-phase+N+PE, 480Vac±10%

Frequency	50Hz/60Hz	60Hz

Backup Output

Rated voltage

3-phase+N+PE, 220/230/380/400Vac

±10% (CE), 480Vac±10% (UL)

3-phase+N+PE, 480Vac±10%

N/A

Current	Max 63A	Max 125A

Frequency	50Hz/60Hz	60Hz

Charger Output

Voltage	3-phase+N+PE, 380/400Vac±10% (CE), 480Vac±10% (UL)

Current	Max 63A	Max 125A

Dimensions (W*D*H)	360mm*560mm*220mm   508mm*220mm*850mm

3-phase+N+PE, 480Vac±10%

Mechanical Parameters

Weight	26kg	57.5kg

*The product is continuously evolving. Please refer to the latest datasheet for specifications.

Web:  www.envictaenergy.com Email:  contact@envictaenergy.com Tel: +1 (408) 304 7181

Add: 675 Sycamore Drive, Ste 150, Milpitas, CA 95035, United States